# COMPUTER NETWORKS

ARYAMAN BAJAJ 24BCE0444                                                                                                   ZANZAN HRUSHAIBH SHRIKRISHNA 24BCE0485                                                     PROF. J. DHEEBA

REVIEW 2 REPORT

# WiFi Signal Mapper

Methodology Review Report

# 1. Introduction and objectives

The project measures WiFi signal strength across the campus and maps it. Each WiFi scan is paired with a GPS position fix. The current dataset holds 12,418 usable measurements from 435 scan points, covering 82 networks over an area of about 420 × 350 m. It was collected on four days between 27 August and 26 October 2026.

The objectives are:

- Collect geotagged received signal strength (RSSI) readings for every visible access point (AP) with low-cost hardware.

- Store the readings in a shared, append-only log that can be reviewed while data is being collected.

- Produce a map that shows per-AP signal strength, per-network coverage and AP density, without overstating the precision of the measurements.

## 2. System overview

The system runs in two stages, joined by a CSV snapshot. The live stage collects readings with the board and a laptop. The offline stage builds the map from the snapshot.

The live stage needs the hardware and access to the Sheet. The offline stage needs only the committed CSV, so anyone can rebuild the map.

## 3. Data acquisition

The hardware is a Seeed XIAO ESP32-C6 microcontroller and a NEO-6M-compatible GPS module. The board's WiFi radio scans for networks in station mode without joining any of them. The GPS is connected over UART at 9600 baud, and the board sends its output to the laptop over USB at 115200 baud.

- Scanning. Scans run asynchronously, back to back. This lets the firmware keep reading GPS sentences (with the TinyGPSPlus library) while the radio hops across channels 1–13. The GPS receive buffer is enlarged to 2048 bytes, so NMEA sentences are not corrupted while a scan is running.

- Geotagging. A fix is used only if it is valid and less than 5 s old. Without a fix the coordinates are written as 0,0.

- Output. Every scan prints one CSV line per visible network: timestamp, latitude, longitude, altitude, satellite count, SSID, RSSI (dBm), channel, encryption (Open/Secured) and a signal-quality label. Networks with a hidden SSID are skipped.

- Quality bands. The label uses fixed RSSI thresholds, and the map uses the same thresholds, so a point's colour always matches the raw log.

| RSSI (dBm) | ≥ −50 | −51 to −60 | −61 to −70 | −71 to −80 | ≤ −81 |
| --- | --- | --- | --- | --- | --- |
| Label | Excellent | Good | Fair | Weak | Very Poor |

Data was collected by walking the area with the rig connected to a laptop. Fixes used a median of 17 satellites (range 4–19).

## 4. Data logging and processing

Logging. A Python logger (wifi_mapper.py) reads the serial stream. It accepts only lines with exactly 10 comma-separated fields and a valid, non-zero fix. It replaces the board timestamp with host time in IST. Accepted rows are batched and appended to a Google Sheet every 5 s, through a service account. When the Sheets API rate-limits a write, the logger retries with exponential backoff, waiting 2 s at first and making up to 5 attempts. The same loop also updates an in-memory state served at localhost:8765, so a live map shows the networks at the current position while collection is running.

Snapshot. The Sheet is exported to data/scan_log.csv, which is committed to the repository. The map is built only from this file, so rebuilding it needs no credentials, serial port or network access.

Cleaning. Rows missing a coordinate, RSSI or SSID are dropped. So are pre-fix rows at 0,0: 212 of the 12,630 logged rows.

Aggregation. build_viewer.py derives three datasets from the cleaned rows:

- AP clusters. For each SSID, DBSCAN with the haversine metric (radius 3 m, minimum 1 sample) merges repeat sightings at the same spot into one point. Each cluster records the median RSSI, so one outlier cannot change its colour. It also records the most common channel and encryption, the number of measurements, and the first and last time seen.

- Coverage grid. Measurements are binned into 5 × 5 m cells for each SSID, and each cell stores its mean RSSI and count. Metres per degree of longitude are calculated once, at the centre latitude of the data.

- Scan points. Rows are grouped by timestamp and position, and each scan point records how many distinct networks were visible there and its strongest RSSI.

The 3 m and 5 m sizes were chosen to match the NEO-6M's accuracy, about 2.5 m CEP. Finer bins would claim more precision than the receiver can deliver.

## 5. Visualisation

The build script puts the three datasets and the Leaflet mapping libraries into one HTML file, viewer/wifi_map.html. The file opens straight from disk, and it needs the network only to load basemap tiles. If the preferred tile provider fails, the viewer tries the next one, and the measurements are plotted even when no tiles load. The map has three views:

| View | Built from | What it shows |
| --- | --- | --- |
| AP dots | AP clusters | One dot per network per location, coloured by median RSSI. The popup lists SSID, signal, channel, encryption, measurement count and first/last seen. |
| Coverage | Coverage grid | Mean RSSI per 5 m cell for one chosen network. An empty cell means no measurement, not no signal. |
| Density | Scan points | A heat layer of how many networks were visible at each point, which shows AP congestion. |

Coverage is drawn as a grid, not a heat blur, on purpose. A heat layer gets brighter where measurements are frequent, so feeding RSSI into one would mostly show where the operator stood longest. Density counts points, which is what a heat layer actually represents, so that view does use one. A side panel filters the AP-dots view by minimum signal, encryption and network, and its statistics update with every filter change.

## 6. Validation and limitations

The data was checked for consistency at three points. Pre-fix rows are filtered both by the logger and again when the map is built. The quality labels written by the firmware use the same thresholds as the map's colour bands. The build script reports how many rows it read, dropped and kept on every run. In the current data the measured RSSI ranges from −97 to −23 dBm, with a median of −79 dBm. Of the 12,418 usable readings, 86% are Weak or Very Poor.

The method has these known limitations:

- Positional accuracy. Resolution is limited by the GPS, at about 2.5 m outdoors. Indoors the receiver keeps its last fix, so indoor readings are less reliable.

- SSID parsing. An SSID that contains a comma splits into 11 fields, so it is dropped. Fixing this needs a change to the serial format.

- Summary statistics. A median shows a cluster's typical value but hides how much the readings vary. The measurement count in each popup shows how many readings a dot summarises.

- Single receiver. Every reading comes from one ESP32-C6 antenna, so the absolute RSSI values depend on this device's sensitivity and how it was oriented.

Possible future work includes a delimiter-safe serial format, a record of GPS accuracy (HDOP) for each fix, and repeated surveys to compare coverage at different times of day.